# Cart-Pole Hermite–Simpson Collocation Solver

This repository contains a MATLAB implementation of a direct collocation optimal control solver for the cart–pole stabilization problem, using the Hermite–Simpson (HS) transcription method and MATLAB’s fmincon nonlinear programming solver.

The method computes an optimal open-loop control trajectory that swings the pendulum from the downward hanging position to the upright position while regulating the cart near the origin.

## 1. Overview

The solver formulates the cart–pole stabilization task as a continuous-time optimal control problem (OCP) and discretizes it using the Hermite–Simpson direct collocation scheme. The result is a large-scale nonlinear programming problem (NLP) that enforces system dynamics through algebraic constraints rather than numerical ODE integration.

This leads to a stable and efficient transcription for highly nonlinear systems like the cart–pole.

## 2. Problem Formulation

### State Vector

The cart–pole system uses a 4D state:

x = [
    x_c;        % cart position
    x_c_dot;    % cart velocity
    theta;      % pendulum angle (0 = downward, pi = upward)
    theta_dot;  % angular velocity
]

### Control Input

u = horizontal force applied to the cart

### Objective

Minimize: 1. Integrated control effort (∫ u² dt) 2. Terminal state error (penalty on deviation from upright stabilized equilibrium)

### Dynamics

The code implements a standard textbook cart–pole model:

theta_ddot and x_c_ddot computed via coupled nonlinear equations

The dynamics are wrapped in the function cartpole_dynamics(x, u) for use inside the collocation constraints.

## 3. Hermite–Simpson Collocation

The time interval [0, T] is divided into N segments. For each segment, the method defines:

- State at knot points: Xₖ, Xₖ₊₁

- State at midpoints: Xₘ

- Control at knot points: uₖ, uₖ₊₁

- Control at midpoint: approximated as (uₖ + uₖ₊₁)/2

At each interval, two constraints enforce the dynamics:

### 1. Midpoint Consistency Constraint

Enforces that the midpoint state matches a cubic Hermite interpolation:

x_m = 0.5*(x_k + x_{k+1}) + (dt/8)*(f_k - f_{k+1})

### 2. Simpson Defect Constraint

Ensures that the change in state matches the integral of the dynamics:

x_{k+1} - x_k = (dt/6) * (f_k + 4*f_m + f_{k+1})

Together, these constraints make the direct collocation transcription third-order accurate and numerically robust.

## 4. Decision Variable Structure

A single stacked decision vector z is passed to fmincon:

z = [X_knots(:); X_mid(:); U_knots(:)]

Number of variables:

nX  = nx*(N+1)   states at knots
nXm = nx*N       states at midpoints
nU  = nu*(N+1)   controls at knots

Bounds on states and control are applied element-wise to z.

## 5. Cost Function

The objective combines: 1. Simpson-integration of u² over all intervals. 2. Terminal penalty enforcing upright stabilization:

x_final_error = [x_c - 0; x_c_dot - 0; wrapToPi(theta - pi); theta_dot]
J = R_u * integral(u^2 dt) + x_final_error' * Qf * x_final_error

The angle error is wrapped using wrapToPi to avoid artificial discontinuities.

## 6. Nonlinear Constraints

nonlcon() returns equality constraints enforcing:

- Midpoint consistency

- HS defect at each interval

- Initial condition x(0) = x0

Total number of constraints:

ceq = nx*(2*N + 1)

## 7. Optimization via fmincon

The solver uses:

Algorithm: SQP
MaxFunctionEvaluations: large for stability
MaxIterations: tuned for convergence of large NLPs

The initial guess is a straight-line interpolation from the downward to the upright configuration.

## 8. Output and Visualization

After optimization, the code:

- Reshapes the decision vector back into state and control trajectories

- Plots:

  - cart position

  - cart velocity

  - pendulum angle

  - control input

- Animates the cart–pole system over time using simple graphics

The animation shows: - cart motion on track - pole motion attached to cart - evolution over time from the optimized control

## 9. How to Run

- Save the file as:

cart_pole_HS_collocation_fixed.m

- Ensure MATLAB Optimization Toolbox is installed.

- Run:

>> cart_pole_HS_collocation_fixed

Execution time depends on the number of intervals (N) and tolerance settings.

## 10. Suggested Adjustments

- Increase N for smoother solutions (more accurate but slower).

- Adjust Qf weights to modify target stabilization precision.

- Modify R_u to tune control aggressiveness.

- Replace the initial guess with a smarter warm start (e.g., LQR-based trajectory).

## 11. References

- Betts, J. “Practical Methods for Optimal Control Using Nonlinear Programming”

- Kelly, M. “An Introduction to Trajectory Optimization: Direct Collocation Methods”

- Standard cart–pole dynamics from classical robotics/control literature

This README provides the necessary understanding to modify or extend the solver for more complex systems, constraints, or objectives.

## 12. SQP Algorithm

What Is the SQP Algorithm?

Sequential Quadratic Programming (SQP) is one of the most widely used algorithms for solving nonlinear programming (NLP) problems of the form:

Minimize
  f(x)
Subject to
  equality constraints: cₑ(x) = 0
  inequality constraints: cᵢ(x) ≤ 0
  bounds: lb ≤ x ≤ ub

SQP solves the nonlinear problem by building and solving a sequence of Quadratic Programming (QP) subproblems.

How SQP Works (Conceptually)

On each iteration k:

- Linearize the constraints
All nonlinear constraints are approximated with first-order Taylor expansions:

- c(x_k + Δx) ≈ c(x_k) + Jc(x_k)*Δx

This creates a linear constraint model.

- Quadratize the Lagrangian
The algorithm builds a quadratic approximation of the Lagrangian function

- L(x, λ) = f(x) + λᵀ c(x)

using:

  - gradient of f(x)

  - Jacobians of constraints

  - an approximation to the Hessian of the Lagrangian (quasi-Newton updates)

- Solve a Quadratic Programming (QP) subproblem
The QP finds a search direction Δx by solving:

- minimize     ∇f(x_k)' Δx + 1/2 Δx' H_k Δx

- subject to   linearized constraints

where
H_k ≈ Hessian of the Lagrangian (updated via BFGS or L-BFGS).

- Perform a line search
The algorithm moves to:

- x_{k+1} = x_k + α Δx

where α is chosen to maintain constraint feasibility and reduce the merit function.

- Update multipliers and Hessian estimate
Quasi-Newton updates improve the second-order model of the problem.

This repeats until optimality, feasibility, and step-size conditions are satisfied.

Why SQP Is a Good Fit for Direct Collocation

SQP is often the best algorithm for problems like your cart-pole direct collocation NLP because:

1. Handles nonlinear equality constraints accurately

Direct collocation imposes hundreds or thousands of equality constraints (defects). SQP handles these robustly.

2. Works well with smooth dynamics

Cart–pole ODEs are smooth and differentiable, ideal for SQP.

3. Excellent for medium-scale NLPs

Collocation NLPs with 1k–20k variables fit well within SQP’s design range.

4. Fast local convergence (superlinear)

Once close to the optimal trajectory, SQP converges rapidly.

5. Compatible with finite-difference or analytic gradients

fmincon computes derivatives automatically unless supplied.

Summary (Short Version)

When you specify 'Algorithm','sqp', MATLAB’s fmincon solves the nonlinear optimal control problem by:

- building local quadratic approximations of the objective + constraints

- solving a QP at each iteration

- updating the solution using quasi-Newton Hessian approximations

- enforcing nonlinear dynamic constraints robustly

SQP is typically the preferred algorithm for optimal control problems transcribed using direct collocation.

## 13. Hermite–Simpson Collocation

1. What Hermite–Simpson Collocation Is

Hermite–Simpson (HS) collocation is a third-order accurate direct transcription method for converting an optimal control problem (OCP) into a finite-dimensional nonlinear programming (NLP) problem.

The idea:

- Represent the state and control trajectories with piecewise cubic polynomials.

- Enforce system dynamics at both knot points (segment boundaries) and midpoints.

- Impose constraints so that the polynomial representation exactly matches the ODE dynamics.

HS is widely used because it is more accurate than trapezoidal collocation and much more stable than single shooting.

2. Structure of the Time Grid

You divide the time horizon [0, T] into N intervals of length dt = T/N.

For each interval k:

- At the knot points:

  - State: x_k, x_{k+1}

  - Control: u_k, u_{k+1}

- At the midpoint:

  - State: x_m

  - Control: typically approximated as u_m = 0.5(u_k + u_{k+1})

Thus, each interval has three state values:

x_k   ---- midpoint ----  x_{k+1}

(x_m)

This is the fundamental grid on which the dynamics constraints are enforced.

3. Polynomial Representation Behind HS

Within each interval [t_k, t_{k+1}], HS assumes that the state trajectory is a cubic polynomial. Given two endpoints and slopes at those endpoints:

- x_k (state at start)

- x_{k+1} (state at end)

- f_k = f(x_k, u_k) (state derivative at start)

- f_{k+1} = f(x_{k+1}, u_{k+1}) (state derivative at end)

There is a unique cubic polynomial that satisfies these four values.

This polynomial serves as the implicit approximation to the true system trajectory.

The midpoint value x_m should lie on this cubic curve—and this is how the midpoint consistency constraint arises.

4. The Two Key Constraints

HS imposes two constraints per interval per state variable:

4.1 Midpoint Consistency Constraint

This enforces that the midpoint of the cubic curve matches the midpoint state variable.

Derived from Hermite interpolation theory, the constraint is:

x_m  =  (x_k + x_{k+1})/2  +  (dt/8)*(f_k - f_{k+1})

This is exact for the cubic interpolant, not an approximation.

Meaning:
Your decision variable x_m must lie on the cubic interpolant defined by the knot states.

4.2 Simpson (Integral) Defect Constraint

This enforces that the integral of the dynamics over the interval matches the cubic approximation.

Derived from Simpson’s rule for integrating ODEs:

x_{k+1} - x_k  =  (dt/6) * ( f_k + 4*f_m + f_{k+1} )

Where
f_m = f(x_m, u_m) is the dynamic evaluated at the midpoint.

This ensures that over each interval, the accumulated state change exactly matches the integrated ODE according to the cubic model.

5. Why Two Constraints?

Because the cubic polynomial needs:

- Two state values

- Two derivative values

The midpoint consistency ensures the cubic curve is internally consistent.
The Simpson defect ensures the derivative integral matches the endpoint difference.

Together, they guarantee:

- Third-order accuracy

- High stability

- Proper enforcement of dynamics

Thus HS is considered a high-quality implicit ODE integrator embedded within the NLP.

6. Decision Variables in HS

For each interval:

| Item | Quantity |
| --- | --- |
| States at knots | N+1 |
| States at midpoints | N |
| Controls at knots | N+1 |

Controls can optionally have midpoint values, but often unnecessary—midpoint controls are interpolated linearly.

All these are stacked into a single NLP vector z.

7. Constraints and Unknowns Count

If there are:

- nx state variables

- N intervals

Then:

- Number of HS constraints = nx * 2*N
(midpoint consistency + defect constraint)

- One additional constraint for initial condition: nx

Total:

nx*(2*N + 1)

This forms a large-scale but structured NLP.

8. Local Accuracy Properties

Hermite–Simpson is:

- 3rd-order accurate in state trajectory

- 3rd-order accurate in control if controls are cubic

- 4th-order accurate if midpoint controls are optimized separately (optional refinement)

- Highly stable vs. forward Euler / trapezoidal methods

This accuracy per interval means you can use fewer intervals than lower-order collocation methods.

9. Why HS Works Well for Optimal Control

1. High accuracy with relatively few nodes

Meaning smaller NLP → faster solve time.

2. Avoids ODE integration

Dynamics are enforced by algebraic constraints, eliminating integration instability.

3. Robust for nonlinear systems

Such as cart–pole, acrobot, inverted pendulum, quadrotors, spacecraft.

4. Direct compatibility with SQP and interior-point methods

HS constraints are differentiable and well-conditioned.

10. Typical Workflow in HS

- Choose number of intervals N

- Initialize knot states X, midpoint states Xm, and controls U

- Build NLP decision vector z

- Implement the HS constraints

- Build a cost function (running + terminal)

- Call an NLP solver (fmincon or IPOPT)

- Extract the optimized state and control trajectories

11. Analytical Summary

Hermite–Simpson is essentially a global, constrained implicit Runge–Kutta method, embedded into an NLP solver:

| Interpretation | Meaning |
| --- | --- |
| Cubic Hermite interpolation | Represent state trajectory smoothly |
| Simpson rule | Enforce ODE consistency via integral constraints |
| Direct collocation | Turn ODE into algebraic constraints |
| Resulting NLP | Solve for trajectory and control simultaneously |