Overview

This code implements an Optimal Control solver for the inverted pendulum using Direct Collocation with the Hermite–Simpson transcription method, solved through fmincon in MATLAB.
The goal is to find a control input trajectory ( u(t) ) that stabilizes a pendulum near the upright configuration while minimizing control effort.

The approach converts a continuous-time optimal control problem into a finite-dimensional nonlinear programming problem (NLP):

[
\min_{x(t),u(t)} \int_0^T u(t)^2 , dt
]

subject to the pendulum dynamics

[
\dot{x} = f(x,u)

\begin{bmatrix}
\dot{\theta} \
\frac{g}{\ell}\sin\theta + \frac{1}{m\ell^2}u
\end{bmatrix},
]

boundary conditions, and discretized dynamic constraints.

Direct Collocation Using Hermite–Simpson

1. Discretization

The time horizon ([0, T]) is divided into ( N ) equal intervals with step size ( h = T/N ).
At each grid point ( k = 0, 1, \dots, N ), the method stores:

- State vector ( x_k = x(t_k) )

- Control ( u_k = u(t_k) )

All states and controls are stacked into one decision vector:

[
z = [x_0, x_1, \dots, x_N, u_0, \dots, u_N].
]

This transforms the infinite-dimensional control problem into a finite-dimensional optimization problem.

2. Hermite–Simpson Collocation Constraint

For each interval ( k ), the solution must satisfy the ODE dynamics using a third-order accurate Simpson integration rule.

The defect constraint is:

[
x_{k+1} - x_k - \frac{h}{6} \left( f_k + 4 f_c + f_{k+1} \right) = 0,
]

where:

- ( f_k = f(x_k, u_k) )

- ( f_{k+1} = f(x_{k+1}, u_{k+1}) )

- The collocation state is computed as:

[
x_c = \frac{x_k + x_{k+1}}{2} + \frac{h}{8}(f_k - f_{k+1}),
]

- The collocation control is:

[
u_c = \frac{u_k + u_{k+1}}{2}.
]

This ensures the trajectory is dynamically consistent across the entire horizon.

Nonlinear Optimization Problem

Objective Function

The cost is the integral of control effort:

[
J = \int_0^T u(t)^2 dt.
]

Under uniform discretization:

[
J = h \sum_{k=0}^{N} u_k^2.
]

The objective gradient is analytic:

[
\frac{\partial J}{\partial u_k} = 2 h u_k.
]

The objective has no dependence on the state variables, so all gradients w.r.t. (x) are zero.

Equality Constraints: Dynamics Defects

For each interval ( k ), Hermite–Simpson produces ( n_x ) equality constraints:

[
g_k(x_k, u_k, x_{k+1}, u_{k+1}) = 0.
]

Collecting all intervals yields a vector of defects used by fmincon as nonlinear equality constraints (ceq).

Analytic Jacobians

To improve solver speed and accuracy, all derivatives are computed analytically.

Dynamics Jacobians

For the inverted pendulum:

[
A = \frac{\partial f}{\partial x}, \qquad B = \frac{\partial f}{\partial u},
]

where:

[
A = \begin{bmatrix}
0 & 1 \
\frac{g}{\ell}\cos\theta & 0
\end{bmatrix},
\quad
B = \begin{bmatrix}
0\
\frac{1}{m\ell^2}
\end{bmatrix}.
]

These are used to evaluate the derivative of each Simpson defect with respect to:

- ( x_k )

- ( x_{k+1} )

- ( u_k )

- ( u_{k+1} )

The Jacobian structure is sparse and block-banded, matching the collocation structure.

Structure of the Codebase

Main script: run_pendulum_opt.m

- Defines model parameters

- Constructs an initial guess

- Sets up fmincon options (including gradients)

- Calls objFun.m and nonlconFun.m

objFun.m

- Computes total cost

- Computes analytic gradient w.r.t. the control variables

nonlconFun.m

- Builds all Hermite–Simpson defect constraints

- Computes full analytic Jacobian of constraints

dynamics.m

- Computes pendulum dynamics ( f(x,u) )

- Computes partial derivatives ( A, B )

unpackDecision.m

- Reshapes the decision vector into state/control matrices

Solution Process

- Initialization
The state and control trajectories are initialized to zero.

- Optimization
fmincon (SQP) iteratively updates the decision vector ( z ) to satisfy all constraints and reduce the objective.

- Final Result
The optimized trajectories ( x(t) ) and ( u(t) ) correspond to a dynamically consistent, feasible solution that minimizes control effort and stabilizes the pendulum.

Why This Approach Works Well

- Hermite–Simpson provides third-order accuracy, making it suitable for stiff dynamics.

- Analytic gradients significantly improve solver performance.

- The problem structure matches fmincon’s strengths (moderate size, smooth nonlinearities).

- No need for integration inside the solver—the ODE constraints are replaced by algebraic relations.

Extensions

This framework can be easily extended to:

- add state or control bounds

- enforce terminal constraints

- support nonlinear cost terms

- move to sparse solvers (e.g., IPOPT)

- switch to automatic differentiation

- use higher-order collocation (Radau/Legendre)

If you want, I can also generate a README.md, system block diagram, or flowchart figure for documentation.